- EF-37* – Wall mounted, corrosion resistant eye/face wash with ABS plastic bowl, ½” IPS Type 316 stainless steel stay-open ball valve. Valve has stainless steel ball and PTFE seals, stainless steel flag handle and PVC wall bracket.  Unit shall have (4) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a PVC-coated brass head assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1792