- EFFR-11* – Wall mounted, freeze resistant eye/face wash with stainless steel bowl, ½” U.S. made three-way self-draining stay-open brass ball valve, powder-coated cast aluminum flag handle and wall bracket.  Valve handle is mounted on extension that projects through wall.  Unit shall have (4) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GFR1724